
Exhibit A
Madan Das Apartments
File Number SPA 18-96 & LLA 18-97
456 & 450 E. Alameda Street

[image: image2.png]



City of Manteca

Community Development Department

Conditions of Approval
3/6/19 
Project File Numbers:
SPA 18-96 & LLA 18-97
Project Name:
Madan Das Apartments 
Project APN:
223-120-070 & 080
Project Address:
456 & 450 E. Alameda St.
This list of conditions is not intended to be all-inclusive or a comprehensive list of City regulations.  All conditions are referenced to the Site Plan (dated ______) on file with the City of Manteca, Community Development Department, Planning Division.  

A. Community Development Department Conditions:

1. This project is approved, per the Site Plan dated (____________) and attachments, and as modified by the following conditions of approval, subject to final review and approval by the City of Manteca.

2. This Site Plan Review approval shall be effectuated within a period of two years from this date and if not effectuated shall expire on Date ______, unless altered and/or extended per State law.  Prior to expiration the owner/developer may apply for an extension, to be approved by the Planning Commission, not to exceed a total of one year from original date of expiration.

3. All conditions of approval shall be satisfied by the owner/developer.  All costs associated with compliance with the conditions shall be at the owner/developer’s expense.
4. This project is subject to the EIR/ MND and Mitigation Monitoring and Reporting Program adopted for the Project Name (SCH # if applicable).
5. Developer shall be responsible for contacting all affected agencies, and for the timely payment of all applicable fees associated with this project.

6. All roof mounted equipment shall be fully screened by the roof parapet.

7. The refuse enclosure shall be finished and painted to match the building.

8. Signage is not approved as part of this application.  The building shall have a maximum of ___ square feet of signage.  A sign plan shall be reviewed by the Planning Department prior to issuance of a Building Permit.

9. The applicant shall provide bicycle parking as required by the Zoning Ordinance.  The location and design of the parking shall be reviewed and approved by the Community Development Department.
10. Landscaping submitted with the improvement plans shall be reviewed and approved by the Parks and Recreation and Community Development Departments. 
11. A minimum of 25% of trees planted on site shall be 24-inch box.

12. Thirty percent of the trees planted on site shall be an evergreen species.

13. A lighting plan shall be submitted for review and approval.  All lighting shall be decorative, and shall maintain 1 foot candlelight with pedestrian walkways and parking lots.
14. The applicant is responsible for contacting all appropriate utility companies to obtain agreements for extension and/or relocation of services necessary for the proposed development.
15. Enhanced paving shall be provided at the project entrance.
16. Backflow preventers shall be screened from view using evergreen plant materials.
17. This approval shall not preclude the application to development of the property of changes in City laws, regulations, plans or policies, the term of which are specifically mandated and required by changes in State or Federal laws or regulations.

18. The applicant shall secure and comply with all applicable federal, state and local licenses, permits, authorizations, conditions, agreements, and orders prior to or during construction and operation, as appropriate. 
19. All conditions of approval for this project shall be written by the project developer on all building permit plan check sets submitted for review and approval. These conditions of approval shall be on, at all times, all grading and construction plans kept on the project site. It is the responsibility of the building developer to ensure that the project contractor is aware of, and abides by, all conditions of approval. Prior approval from the Planning Manager must be received before any changes are constituted in site design, grading, building design, building colors or materials, etc. 

20. Should the project be found, at any time, not to be in compliance with any of the Conditions of Approval, or should the applicant construct or operate this development in any way other than specified in the Application or Supporting documents or presentations to staff, Planning Commission or City Council, as modified by the Conditions of this Approval, then the terms of this Approval shall be considered to be violated. 
21. With respect to any claim, action of proceeding against the City, its officials, employees or agents relating to the action or inaction of the City in reviewing, approving or denying entitlements of any type, the Developer agrees to indemnify, hold harmless and defend the City and its elected and appointed councils, boards, commissions, officers, agents, employees, and representatives from any and all claims, costs, and liability for claims of damage, for any property damage or personal injury, including death, which may arise as a result of any negligent acts or omissions by Developer or Developer’s contractors, subcontractors, agents, or employees in connection with the construction, improvement, or operation, of the Project. Developer agrees to indemnify, hold harmless and defend the City and its officers, agents, employees, and representatives from any and all actions for damages caused or alleged to have been caused by Developer’s activities in connection with the Project. This Agreement applies to all damages and claims for damages suffered or alleged to have been suffered arising out of or in connection to any and all Project operations, regardless of whether or not the City prepared, supplied or approved plans or specifications or both for the Project. 
22. All activities undertaken in accordance with this approval shall comply with the City’s General Plan and Municipal Code. In cases of conflict between the City’s Municipal Code or map-specific conditions of approval, the governing priority shall be, to the extent legally permitted, as follows: 1) Municipal Code regulations; 2) project-specific conditions; 3) standard conditions. 
23. The developer shall pay all applicable processing fees, permit fees, City development fees, fire fees, school fees, drainage fees, habitat conservation fees and other public entity fees in effect at the time of the issuance of the applicable permit.  Developer shall be responsible for contacting all affected agencies, and for the timely payment of all applicable fees associated with this project. 
24. Approval of this application does not constitute approval of any other entitlement or any other necessary permit, license, or approval.

B. Engineering Division:
1. All improvements shall comply with the City of Manteca Standard Plans and Specifications. Improvement plans shall be submitted to the City Engineer for approval. An encroachment permit is required for all work within the public right-of-way.

2. Developer shall provide easements, requested by the respective utility companies, within the subdivision and shall show said easements on the final subdivision map. Any existing facilities within or adjacent to the Project that are affected by this Project shall be relocated and placed underground at the Developer’s expense.

3. Developer shall dedicate ten-foot (10’) wide public utility easements on all street frontages for underground facilities and appurtenances.

4. Developer shall indicate on the improvement plans topographical information which shall include one-foot (1’) contour intervals and benchmark data based on City datum.

5. During all construction phases, Developer shall comply with City Laws regarding dust control. Developer shall also comply with San Joaquin Valley Unified Air Pollution Control District Regulation VIII (Fugitive Dust Prohibitions) in an effort to reduce the amount of fine particulate matter (PM10) entrained into the ambient air from man-made sources. 

6. Prior to or with the Building Permit plan set the following shall be submitted. Review and approval of these items by the Community Development Department must occur prior to or with the issuance of the first building permit associated with this project.

a. On-site grading and drainage plan,

b. On-site utility (sanitary sewer, water and storm drain) plan,

c. Joint Trench plans,

d. Erosion control plans,

e. Dedication of required rights-of-way and granting of all required easements for public utilities,

f. Stormwater Pollution Prevention Plan (SWPPP), and

g. Documentation, as required in the Post-Construction Stormwater Standards Manual, showing compliance with WQO NPDES 2013-0001-DWQ.

The plans specified in (a) and (b) above shall be prepared by a Registered Civil Engineer. 

The items in (f) and (g) above shall be prepared by a Qualified SWPPP Developer (QSD).

7. Joint trench utility installation shall be in accordance with Manteca Municipal Code Chapter 13.34 and City Standards.

8. All address numbers shall be plainly visible from the street fronting the property. Said numbers/letters shall contrast with background.
9. City easement dedications shall be completed as a condition of the first final inspection of a building permit for this development. This shall be noted on the cover of the building permit submittal.

Site

10. On-site parking area pavement surface drainage slope shall be as follows:  Minimum cross-slope on asphalt concrete is 2% or concrete is 1 % with a maximum of 5%. Concrete valley or curb gutters shall have a minimum slope of 0.25%.  The pavement slope in ADA areas shall be 1.5% minimum to 2% maximum.

11. Provide City Standard Refuse/Recycling Enclosures on-site at the locations shown on the site plan. Sizing shall be in accordance with City Standards. 

12. The refuse enclosure shall be graded so there is no storm drain or other flow run-on or run-off from the enclosure area. The enclosure area shall have a connection to the sanitary sewer system or a connection to the storm system, in compliance with the City’s Post-Construction Manual.

13. On-site curbing shall conform to City of Manteca Standard ST-35, "Parking Area Curbs”. 

14. Developer shall ensure no buildings are constructed across property lines.

15. Developer shall dedicate an emergency vehicle access easement, to the City of Manteca, over the twenty-six foot (26’) wide drive aisle and the emergency vehicle turnaround area.

16. Landscape planting at driveway entrances/exits shall be maintained to a maximum height of three and one-half feet (3.5’).

17. Developer shall sign and stripe both sides of the drive aisle and the emergency vehicle turnaround area as No Parking.
18. Developer shall complete a lot line adjustment to merge the properties covered by this site plan application. The lot line adjustment shall record prior to the issuance of a building permit for this project.

Streets

19. The existing curb and gutter along the street frontages of this project will be inspected prior to the final inspection of the site work building permit. The Developer shall remove and replace damaged or dilapidated sections of the curb and gutter, as instructed by the City.

20. The thickness of all sidewalks installed with the project shall be six inches (6”).

Water

21. Improvements shall be constructed in conformance with the latest version of the City Water Master Plan.

22. A minimum ten foot (10’) separation, from outside of pipe to outside of pipe, shall be maintained between water mains and parallel sanitary sewer, storm drain, and irrigation lines.

23. Existing wells within the boundary of the proposed development which are not approved for use by the City, shall be abandoned in accordance with San Joaquin County Public Health Services requirements. 

Storm Drainage

24. Improvements shall be constructed in conformance with the latest edition of the Storm Drain Master Plan, the City’s Post-Construction Manual and City Standards.

25. A preliminary storm drainage plan shall be submitted to the City Engineer for approval prior to final improvement plan submittal. The plan shall be accompanied by calculations for peak flows, total runoff, pipe sizes, detention basin volume and evidence of groundwater depth.

26. This project shall submit calculations, per Chapter 3, Section 3.1 of the City’s Storm Drain Master Plan, showing attenuation of the storm drain flows for this project is not required.

27. All drain inlets shall be marked "No Dumping - Drains to River". Drain markers shall be purchased from the City of Manteca at cost plus 15% administrative charge, and installed by the Developer prior to acceptance of the improvements.

28. Project applicant shall incorporate appropriate site design measure(s) and submit the results of the Post-Construction Runoff Standards Manual. The City of Manteca approval of the proposed measures is precedent to issuance of any building, grading or construction permits.

29. Project applicant shall develop and submit a Project Stormwater Plan that identifies the methods to be employed to reduce or eliminate stormwater pollutant discharges through the construction, operation and maintenance of source control measures, low impact development design, site design measures, stormwater treatment control measures, and hydromodification control measures. Design and sizing requirements shall comply with the 2015 Post- Construction Stormwater Standards Manual. City of Manteca approval of the Project Stormwater Plan is precedent to issuance of any building, grading, or construction permits. Two paper copies and an electronic copy of the Project Stormwater Plan shall be provided to the City of Manteca

30. Project applicant shall develop a hydromodification management plan to ensure the post-project stormwater runoff flow rate shall not exceed estimated pre-project flow rate for the 2-year, 24-hour storm. The hydromodification management plan shall be incorporated into the Project Stormwater Plan.

31. Project applicant shall develop and submit an Operations and Maintenance Plan that identifies the operations, maintenance, and inspection requirements of all stormwater treatment and baseline hydromodification control measures identified in the approved Project Stormwater Plan. City of Manteca approval of the preliminary Operations and Maintenance Plan is precedent to issuance of any building, grading, or construction permits. Two paper copies and an electronic copy of the Maintenance Plan shall be provided to the City of Manteca.

32. City of Manteca approval of the final Operations and Maintenance Plan and recordation of the Maintenance Access Agreement is precedent to first building final inspection for this project. Two paper copies and an electronic copy of the final Operations and Maintenance Plan shall be provided to the City of Manteca.

33. Post-Construction Management Practices shall conform to the City’s adopted Multi-Agency Post Construction Stormwater Standards Manual.

34. Where conflict between standards arises, the standard most-protective to water quality, to public health and safety, and against flooding shall be utilized.

35. Prior to any land disturbing construction activities occurring on a project, Developer shall meet the requirements of NPDES. For sites exceeding 1 acre of disturbance area that are deemed non-exempt, contractor shall prepare and submit a Storm Water Pollution Prevention Plan (SWPPP) and apply for a permit under the California General Construction NPDES permit. SWPPP shall be prepared and signed by a Qualified SWPPP Developer (QSD) certified by the State Water Resources Control Board. All modifications to SWPPP shall be implemented by a QSD in responsible charge for the project. The SWPPP shall be implemented under the supervision of a Qualified SWPPP Practitioner (QSP). For permit information, contact the State Water Resources Control Board (SWRCB) at:

State Water Resources Control Board
PO Box 1977, Sacramento, CA 95812-1977
Attn: Storm Water Permitting Section
Telephone: (916) 341-5537

To log in to the SWRCB Storm Water Multiple Application and Report Tracking System (SMARTS) to enter site information and apply for permit, please contact the City of Manteca Public Works Department to establish a project and authorize data entry access.

All other sites shall conform to the City of Manteca Standards, the California Green Building Standards, and Section E.10 of the NPDES permit 2013-0001-DWQ. All construction involving land disturbing activities shall submit for approval an Erosion Control and Sedimentation Plan (ESCP) prepared and signed by a QSD. All ESCP treatment measures and BMPs must be maintained at all times until construction is completed and the site is stabilized as defined under the Construction General NPDES permit.

Prior to issuance of the first grading or building permit for a project, a copy of the SWPPP or ESCP shall be submitted by the developer and approved by Authorized Signatory or Legally Responsible Person (LRP) for the City’s NPDES program. Contact the City of Manteca Public Works Department to identify appropriate person for review and approval of plans and documents.

36. It is recognized that the design and calculations which have been submitted thus far to demonstrate this project’s compliance with the City’s Post-Construction Stormwater Standards Manual is approved for project entitlements but may require further refinement for final approval, which is precedent to issuance of any building, grading, or construction permits.

37. Applicant shall pay fair-share costs for storm drainage improvements, including improvements downstream to improve capacity or water quality treatments needed to conform to current Storm Drain Master Plan, City standards and support the development.

38. Applicant shall complete the CDD development memorandum, required by Storm Drainage Agreement Amendment No. 1, and submit it to SSJID for review.

Sanitary Sewer

39. Improvements shall be constructed in conformance with the latest version of the City Wastewater Collection System Master Plan and City Standards.

40. A cleanout shall be installed on the existing sanitary sewer line, adjacent to City right-of-way, where the sanitary sewer line enters the development. This cleanout shall be the end of the City’s maintenance responsibility for the sanitary sewer system.

41. Any existing septic tank(s) on the property that will not be approved by the City shall be abandoned in accordance with the permitting requirements of the San Joaquin County Environmental Health Department prior to issuance of the first building permit.

42. A preliminary sewer plan shall be submitted to the City Engineer for approval prior to final improvement plan submittal. The plan shall be accompanied by calculations for peak wet weather flows showing pipe sizes and slopes for the entire development.

C. Parks & Recreation Department:
1.

D. Fire Department:
36. Site Address:  Building address numbers shall be plainly visible from the street fronting the property.  Said building address numbers shall contrast with their background and be a minimum of 12” in height

a. Building Numbers for rear buildings shall be visible from Main drive aisle.  Building numbers will be placed in a uniform position on the building façade.  

b. Individual units shall be numbered consecutively according to their floor. All units on the first floor shall be in the 100 series, second floor in the 200 series and third floor units in the 300 series. 
c. Each building shall have a durable, all-weather sign listing the building designator and the units contained within. Example:  

                

[image: image1.wmf]                                Bldg. A                       -or-            Bldg. 2

                        Units 101-106                                                                      Units 107-113
                     201-206                                                                               207-213

                                      301-306                                                                               307-313

37. Fire Department Access:  

a. Provide a diagram showing a minimum of 44’ external radius and internal radius of 25’ on all main entries, drive aisle and parking lot areas.  The main drive aisle shall be designated Emergency Vehicle Access Lane and be a minimum of 26’ wide.  

b. A diagram showing proposed “Fire Lane” shall be submitted for approval to the Office of the Fire Marshal.  Fire Lanes shall be marked in accordance with California Fire Code (Appendix D103.6 SIGNS and Manteca Fire Code 15.24    

38. Fire Hydrants:  Plans and specifications for fire hydrant systems shall be submitted for review and approval prior to construction. 

a. Fire hydrants shall be installed on a minimum 8-inch looped water main. 

b. Fire hydrants shall be installed around all buildings at a maximum spacing of 300 feet in accordance with the City of Manteca Standards and California Fire Code.  Corners should be utilized for hydrant locations when possible.

c. Hydrants subject to vehicular damage shall be protected in an approved manner and not be obstructed by vehicles or other obstructions. 

d. Provide 15 feet of clearance on either side of fire hydrant from stopped or parked vehicles. 

e. Additional fire hydrants may be required to meet the minimum spacing requirement of hydrants within 90 feet of Fire Department Connections, (FDC) for Fire Sprinkler Systems.

f. The on-site fire protection system (fire hydrants and associated water mains) shall be installed prior to the issuance of building permits.  

39. Underground piping for the fire sprinkler shall be approved by the Fire Department prior to permit issuance.    

40. All above ground gas meter, regulators, and piping exposed to vehicular damage shall be protected in an approved manner.

Fire Suppression Systems:  All buildings must be protected by an approved monitored automatic sprinkler system in accordance to NFPA 13, CFC, CBC, and Manteca Municipal Code Section 15.24.  

a. Fire Alarm/Sprinkler monitoring shall be point ID.  

41. Fire Suppression Approval.  Plans and specifications for fire suppression systems shall be submitted for review and approval prior to construction.  If work differs from approved plans, a set of “as built” shall be submitted to the Fire Department prior to final inspection.  

42. Plans proposed to be listed as deferred submittals shall be clearly labeled on the cover building permit plan submittal cover sheet.  

43. Inspections.  Fire protection systems (fire hydrants, water mains, etc.) shall be installed, tested and approved by the City prior to the issuance of any building permits.  The Fire Department shall witness all system and acceptance tests.  Please provide 24-hour notice prior to any system or acceptance tests.  209-456-8340 
44. A Fire Department approved “key lock box” shall be properly installed near the main entrance.  A 3200 series lock box can be order online directly from KNOXBOX.COM.  Contact the Office of the Fire Marshal at FireMarshal@mantecafire.org for additional information.  

45. All above ground gas meters, regulators, and piping exposed to vehicular damage shall be protected in an approved manner.

46. Trash enclosures/dumpsters shall not be allowed within 5 feet of any combustible walls, openings, or combustible roof eaves, unless protected by an automatic fire sprinklers.

E. Building Division:
1. Accessible parking spaces complying with CBC § 11B-502 shall be provided in accordance with CBC Table 11B-208.2 and van accessible stalls complying with CBC §11B-208.2.4.  These parking space(s) shall be located as close as possible to the primary entrance to the building.  CBC § 11B-208.2, § 11B-208.2.4 and § 11B-208.3.1.  

· Please show the total number of parking spaces (Existing and New), accessible parking stalls and van accessible parking stalls on the site plan.

2. Accessible routes shall be provided per CBC § 11B-206.  At least one accessible route shall be provided within the site from accessible parking spaces and accessible passenger loading zone; public streets and sidewalks; and public transportation stops to the accessible building or facility entrances they serve.  Where more than one route is provided, all routes must be accessible.  CBC § 11B-206.2.1.

3. At least one accessible route shall connect accessible buildings, accessible facilities, accessible elements, and accessible spaces that are on the same site.  CBC §11B-206.2.2.

4. Applicant shall provide a complete and accurate site plan with appropriate dimensions showing compliance with CBC 602 and 708 for separation distances, allowable openings, distance to property lines, etc. as the current plan lacks necessary information for review that may affect the proposed plan.

5. The proposed addition shall meet the type of construction of the existing building and shall meet the allowable building area as required per 2016 CBC.  Provide the allowable building area calculation on the cover sheet or site plan.

6. All entrances and exterior ground-floor exits to buildings and facilities shall be accessible and shall comply with CBC § 11B-404.  CBC § 11B-206.4.1.  

7. Developer shall provide a minimum of a one-hour fire rated wall at the location where any fire separation distance of less than5 feet occurs in accordance with the requirements of the 2016 California Building Code and all openings shall be adequately protected per the 2016 California Building Code requirements.  Please note that any existing structures will have significant alterations required to meet the requirements of the building code.  Please verify compliance on the plans.

8. Adequate sanitary facilities shall be provided for the facility, per the requirements of CPC Table 422.1 and Chapter 4 of the California Plumbing Code.

· Pursuant to Section 422.4 of the California Plumbing Code, toilet facilities shall be accessible to employees at all times, should not be more than 500 feet from where employees are regularly employed.  The plans shall indicate the location of the toilet facilities and the travel distance from work areas.

9. Designated parking for clean air vehicles shall comply with 2016 CGBSC §5.106.5.2 and Table 5.106.5.2 (8%) and Electric Vehicle Charging spaces shall comply with 2016 CGBSC §5.106.5.3 and Table 5.106.5.3.3 (6%).

10. Provide long-term secured bicycle parking for additions or alterations that add 10 or more parking spaces per CGBSC §5.106.4.

11. Developer shall comply with all requirements of the Model Water Efficient Landscape Ordinance (MWELO)and all plans shall be submitted and approved prior to permit issuance.

12. For new buildings 10,000 square feet and over, building commissioning shall be included in the design and construction processes of the building project to verify that building systems and components meets the owner’s or owner representative’s project requirements.  Commissioning shall be performed in accordance with 2016 CGBSC §5.410.2 by trained personnel with experience on projects of comparable size and complexity.  Commissioning requirements shall include the following and shall be included in the construction documents

a. The owner’s or owner representative’s project requirements.

b. Basis of Design

c. Commissioning measures shown in the construction documents

d. Commissioning plan

e. Functional performance testing

f. Documentation and training

g. Commissioning report. 

13. At time of building permit submittal the developer shall incorporate all Conditions of Approvals from all departments and imprint into the submittal set of construction documents/plans.

F. Police Department:

ADVISORY NOTES:

Requirements from outside agencies:

A. SSJID

B. San Joaquin County Multi-Species Habitat Conservation & Open Space Plan (SJMSCP):

C. San Joaquin County Environmental Health Department (EHD):

D. Manteca Unified School District:
Developer shall contact Manteca Unified School District Facilities Planning regarding school fees and requirements and shall provide proof of payment or waiver of such fees to the Community Development Department.

E. San Joaquin Valley Air Pollution Control District
The developer shall comply with all San Joaquin Valley Unified Air Pollution Control District polices, rules and regulations.  

F. San Joaquin Council of Governments

	Dept. Review Approval Sheet 
	
	
	
	

	Dept
	Initial 
	Date
	Date
	Date
	Final

	Engineering
	
	
	
	
	

	Parks & Rec
	
	
	
	
	

	Fire
	
	
	
	
	

	Police
	
	
	
	
	

	Building
	
	
	
	
	

	Planning
	
	
	
	
	


Conditions of Approval 3/6/19
Page 8 of 10

